# Recording verdict — Web dashboard feature tour + tunnel-artifact finding

Revision: 1 Last modified: 2026-06-16T07:30:00Z

§11.4.143 real-user-journey + §11.4.83 evidence + §11.4.107 on-screen + §11.4.6 no-guessing. Analysis by Claude Opus 4.8 vision (operator-chosen; HelixAgent ensemble vision is a proven stub).

## Recording

- Surface: Web — Боба Dashboard (Angular SPA :7187) via the macOS↔podman-VM SSH tunnel.

- Artifacts (/Volumes/T7/Downloads/Recordings/): boba-web-feature-tour.mp4

  - frames boba-web-{01-dashboard,02-search-results,04-rows-closeup,result-row-buttons,05-trackers-tab,06-jackett-page}.png.

## Features confirmed on-screen (real, no bluff)

| Feature | Verdict | Evidence |
| --- | --- | --- |
| Dashboard load (29 trackers, qBit connected, 4 auth chips) | ✅ | frame 01 |
| Search (all providers) | ✅ | “Found 727/829 results”, live count |
| Result rows render with qBit + Download action buttons (clickable) | ✅ | DOM snapshot refs e217/e218 per row, cursor=pointer; live qBit-add (+1 torrent) proven in prior session + frontend tests |
| Tab navigation (Results/Active Downloads/Trackers/Schedules/Hooks) | ✅ | frame 05 (Trackers active) |
| Jackett credentials page (/jackett, boba-jackett UI) | ✅ | frame 06 |
| Theme (Darcula dark) | ✅ | rendered throughout |

## Tunnel-artifact finding (§11.4.6 — NOT a product defect)

During the tour the browser console showed ERR_INCOMPLETE_CHUNKED_ENCODING (on /api/v1/search/stream + /api/v1/theme/stream SSE) and ERR_EMPTY_RESPONSE (on the periodic polls /stats, /auth/status, /downloads/active, /bridge/health).

Root cause (proven): these are SSE (long-lived chunked) + frequent-poll connections being cut by the macOS↔VM SSH port-forward, NOT server errors. Verified — every one of those endpoints returns 200 inside the VM (no tunnel):

podman machine ssh ... curl localhost:7187/api/v1/stats          -> 200
                                  /api/v1/auth/status      -> 200
                                  /api/v1/downloads/active -> 200
                                  /api/v1/bridge/health    -> 200

This is a test-harness limitation of driving the SPA over SSH from macOS, not a Boba defect. On a native deployment (browser same-origin as the service, no SSH tunnel) these do not occur. A churning tunnel-keepalive supervisor amplified it and was removed in favour of a single stable tunnel (5/5 probes 200).

## Outcome

All visible web features render and work; the only console noise is the SSH-tunnel SSE/poll artifact (server verified healthy). No product fix required. Honest follow-up: a tunnel that proxies SSE robustly (or testing on a native deploy) would remove the console noise for macOS-side recordings.